Streptavidin-functionalized MSP

|  | Product Specifications and Features |
| --- | --- |
| Product Name | Streptavidin-functionalized MSP (FBD-MSP-ZSAV) |
| Manufacturer | Chongqing FBD Biotechnology Co. Ltd., China (http://www.fbdbio.com) |
| Structural properties | covalent conjugates through amide bond between streptavidin and FBD-MSP-ZCOOH (paramagnetic noncore Fe3O4, polymer coats bearing zwitterions at pH from 3 to 12, physical diameter of ~0.6m in vacuum while aqueous diameter of ~1.0m), a long linear arm between MSP and streptavidin, no coating of surface with albumin |
| Performance | good suspension stability, rapid response to external magnetic fields, negligible nonspecific adsorption of native proteins; binding equilibrium in ~5.0 min; large binding capacity for biotinylated substances |
| Applications | binding of biotinylated substances to separate interacting components |
| Binding capacity | the isolation of 1.6 mg biotinylated calf intestinal alkaline phosphatase by 1.0 g MSP; saturated binding of ~14 mg biotinylated calf intestinal alkaline phosphatase and of ~10 mg biotinylated antibodies, by 1.0 g MSP |
| Package quantity | FBD-MSP-ZSAV, 30 g/L; see labels and signs on package |
| Shelf-life | > 18 months at 2~8 0C when closed and kept in dark |
| Storage/shipment | both long-duration storage and shipment at 2~8 0C |
| Attention | no freezing or acidification; avoiding any treatment causing aggregation |
|  | Recommendation for operations |
| Conditions | binding reaction for 10 min at neutral pH and ambient temperature |
| Usage fields | smaller quantity than others to isolate the same biotinylated substances |
| Optimization | smaller quantity provided higher sensitivity for chemiluminescence assay |
|  | Safety and Disposal |
| Toxicity | sodium azide in the original suspension; low toxicity of other substances |
| Warning | no swallowing or contact with skin; far away from children; no eating/drinking |
| Emergence | washing skin with lots of water in case of contact; see doctors immediately |
| Environment | no toxicity of the product itself; toxicity of suspension mixture |
| Final Disposal | collection of suspension for treatment; following requirement with product |
| Filed date | 20170616 |